# 3D-Tag

General characteristics

- Objects materials: plastic, paper

- Dimensions: 50 х 50 ± 0,5 mm

- Antenna dimensions: 44 х 44 mm

- Pitch: 52,7 mm

- Standard: ISO 18000-63 / EPC global Gen2v2

- Operating frequency: 865 – 870 MHz

- Readrange: 10 m

- Operating temperature: - 20°С to +80°С

- Operating humidity: no more than 80 %

Electrical specifications

| Memory | IC |  |
| --- | --- | --- |
|  | NXP UCODE 8 | NXP UCODE 8m |
| EPC | 128 bits | 96 bits |
| Unique TID | 96 bits | 96 bits |
| Access Password | 32 bits | 32 bits |
| Kill Password | 32 bits | 32 bits |
| User memory | - | 32 bits |

The structure of the tag

- Top material: PEFC RAFLACOAT PLUS

Delivery format

- Delivery format on the basis of self-adhesive

- Number of labels per roll: 2 000 pcs

- Inner roll diameter: 76 mm

- Outer roll diameter: not more than 200 mm

- MOQ: 1 roll

Directional diagram

XZ-plane

YZ-plane

The normalized distance of the check, m